| DRY-GUD-039 | Dairy |
| --- | --- |
| Revision: 1.0 | In-Line Sampler Training |
| Approved: 07/22/2026 | Wis. Stat. and/or Wis. Admin. Code: ATCP 82.12 |

SUBJECT: To assist dairy section staff and industry personnel in developing and implementing In-Line sampler training programs.

SCOPE: To help guide DATCP and dairy industry personnel on required materials, documents, frequency of training farm staff on proper handling, use of in-line samplers, and sample collection.

GUIDANCE:

It is the responsibility of the licensed Bulk Milk Weigher and Sampler (BMWS) to train all designated employees on how to properly use an in-sampler and handle any samples collected.

Required Topics:

- Proper hand washing/hygiene.

  - Wash with soap and warm water for a minimum of 20 seconds.

  - Dry hands with single-use disposable towels.

- How to set up device based on manufacturer's specifications.

  - Device assembly.

  - Change septum/insert needle/use peristaltic pump.

  - Attach sample container.

- How to turn device on, set sampling rate.

- How to stop sample collection, when to pause sample collection.

  - Clean-in-place (CIP).

  - Unplanned downtime during milking.

- Proper sample splitting.

  - Follow aseptic handling practices.

    - Sanitize all equipment utilized in sample splitting.

    - Splitting is done in a clean, uncluttered environment.

  - Sample collection container is not more than ¾ full.

  - Sample is between 32°F and 40°F.

    - Collect a temperature control sample.

  - Proper agitation.

    - Roll sample collection container 180° upside down 25 times.

- Proper cleaning of sampler.

  - Follow manufacturer’s specifications.

  - Single-use items vs multi-use items.

    - Single-use items are discarded after one use.

    - Multi-use items need to be properly cleaned and sanitized.

  - Replace wearable parts/pieces.

    - Ensure preventive maintenance schedule is followed based on manufacturer's specifications.

- Proper disassembly.

  - Follow manufacturer's specifications.

- How to fill out sample documentation.

  - Time, temperature, date, patron ID, initials of BMWS or trained employee, trailer ID (Grade A permit number).

  - If you split the sample, include a properly labeled TC.

- Definition of representative sample.

  - A small, accurately selected quantity of milk that reflects the whole milk shipment.

- Knowledge Test.

  - Must include all the above-mentioned materials.

  - Open note is acceptable.

Required Documentation

- SOP for sample device/sample handling.

  - Can come from the manufacturer, if available.

- Device manual.

- Manufacturer’s SOP for preventive maintenance.

- List of trained employees, including unlicensed individuals trained by a licensed BMWS under ATCP 82.12(2)(cs).

- Training log and documentation of training.

  - Training must occur at a minimum of once a year and cover all required materials listed above.

- Completed knowledge test of each trained employee.

- Temperature log for refrigeration.

- Weight tickets.

- Thermometer calibration records.

  - Completed every 6 months; meets ATCP 82.10(2)(e) requirements.

NOTE: In-line sampler manufacturers have training and standard operating procedures available on their websites for reference and use by the BMWS.

REFERENCES

- ATCP 82

- ATCP 65

- DRY-GUD-038 In-Line Samplers on Dairy Farms

- DRY-PIN-028 In-Line Sampler Licensing Inspection Procedures

DOCUMENT HISTORY

The most recent changes to this controlled document are listed at the top of the table

| Revision | Author | Change Description | Approval Date |
| --- | --- | --- | --- |
| 1.0 | Sampling and Transport Team | New document. | 07/22/2026 |

APPROVAL