| DRY-GUD-038 | Dairy |
| --- | --- |
| Revision: 1.0 | In-Line Samplers on Dairy Farms |
| Approved: 07/22/2026 | Wis. Stat. and/or Wis. Admin. Code: ATCP 82.12 |

SUBJECT: To assist industry personnel to ensure a proper DATCP approved device is used and to guide dairy staff on proper use of in-line sampling device.

SCOPE: To help guide DATCP and dairy industry personnel on practical installation, maintenance, and usage of an in-line sampler to align with state and federal regulations.

DEFINITIONS:

- FDA M-b; Memoranda of Milk Ordinance Equipment Compliance: These documents are part of the FDA’s Milk Safety Cooperative Program and provide guidance on equipment compliance. Equipment with an FDA M-b have been reviewed and accepted by the FDA.

GUIDANCE:

Equipment Installation

- Prior to installation, ensure in-line sampler has proper approvals including FDA M-b’s.

  - FDA M-b’s can be found at: https://gams.fda.gov.

- Submit the Application for Milk Handling Equipment and Facility Construction and the Supplemental Application for In-Line Sampler to the Department for review.

  - Applications can be found on the DATCP website: https://datcp.wi.gov.

    - Directions for submission are noted on each application.

  - Provide a P&ID or drawing of the planned installation.

- When planning the installation, consider the following:

  - Review and follow Manufacturers’ Instructions.

  - Location:

    - Device should be installed on a pipeline where the milk is fully homogenous (i.e. installed in way that does not create a dead end).

    - Consider installing on a pipeline that can be empty between milkings.

    - Install downstream of any pre-cooling to help maintain sample temperatures.

    - Location should be convenient for cleaning and maintaining the sampler and sampler components.

  - Collection Container:

    - Install device with a properly sized collection container.

    - If using a rigid collection container, container needs to remain in a vertical orientation.

    - Ensure enough room for container to be maintained at proper sampling temperatures (i.e. under refrigeration).

  - Flow Rate:

    - When setting up the in-line sampler, ensure the rate of sample collection correlates with milk flow rate to ensure a representative sample for each load-out.

- In-Line Sampling devices are not to be modified.

Personnel Daily Handling Practices

- Follow Manufacturer’s instructions.

- Hands need to be washed before handling the sample device or sample container.

- Ensure proper cleaning and maintenance by disassembling sampling device routinely. Inspect and replace consumable components (needle, septum, O-ring, etc.) as required by manufacturer instructions or when signs of wear appear.

- When reassembling, ensure proper lubrication when required.

  - Ensure food grade lubricate is used.

- Ensure sample cooling or refrigeration does not contaminate sample.

  - If using ice, ensure water level does not rise above where the collection container connects to the sampling device.

  - Refrigeration or cooling method

    - Must not freeze any portion of the sample.

    - Needs to be adequate to maintain sample between 32°F and 40°F.

    - Shall only contain the sample and a calibrated thermometer.

- Storage of replacement components (sample container, septum, needle, O-ring, etc.) should be clean, sanitary and organized.

  - Replacement sample containers should be stored in a well maintained, cleanable location or sealed container.

  - Storage space should be dedicated to sampling equipment only.

- Single service components need to be discarded after one use.

- Multi-use sample containers must be replaced when scoring, dis-coloration or other damage occurs. Multi-use containers must be thoroughly cleaned and sanitized after each use.

- Splitting samples shall be done on a clean, uncluttered surface or table.

- If samples are split on farm, 3 sample vials shall be taken: TC, Official, and Antibiotic (App N).

Training and Records

- At least one person on farm shall be a licensed bulk milk weigher and sampler (BMWS).

- The designated BMWS is responsible for creating, implementing and maintaining training and training records.

  - Training records shall be maintained in an organized manner and readily available during inspections.

  - Records should be easy to follow and legible.

- All required documentation must be available and accessible during inspections.

  - Documentation will be reviewed by DATCP staff during routine inspections of Dairy Plants and BMWS.

  - If documentation is incomplete, does not meet the requirements set by DATCP, or is unavailable, it may result in a violation of the licensee.

- Training materials need to include all DATCP required topics.

  - Materials should be sufficient, straightforward, and easy to understand.

  - Training materials can include things like demonstrations, quizzes, PowerPoints or any other means of providing information in an efficient way.

    - If using written materials, including quizzes or worksheets, completed materials need to be maintained for inspection.

- Sample collection records need to be sufficiently completed to track samples for each tanker/load.

REFERENCES

- ATCP 82

- ATCP 65

- DRY-PIN-28

DOCUMENT HISTORY

The most recent changes to this controlled document are listed at the top of the table

| Revision | Author | Change Description | Approval Date |
| --- | --- | --- | --- |
| 1.0 | Sampling and Transport Team | New document. | 07/22/2026 |

Approval